PREWIRED MAINS CONSUMER UNIT

INSTALLATION GUIDE

INSTALLATION SHOULD ONLY BE CARRIED OUT BY A COMPETENT PERSON

- INSTALLATION SHOULD ONLY BE CARRIED OUT BY A COMPETENT PERSON

Installation Guide for “Power Part” Pre-Wired Mains Consumer Unit

Congratulations with your purchase of the “power Part” Pre-Wired consumer unit kit for caravans, motor homes and campervans.

Before installation please check the contents list.

Contents:

- Pre-Wired Consumer unit consisting of:

- 25 Amp, 30mA Residual Current Devise

- 16-amp MCB (Double pole type C)

- 5 Module enclosure

- 4 x 230V output connections (Red VLP-03)

- Earthing Wire

- 1 x Mains inlet connection (t-shaped three way)

- 5 x Easy connect Enclosures (4 x light grey, 1 x dark grey)

5 Modular Enclosure

RCD

MCB

Earth Wire

Mains Inlet Connection

4 x 230V Outputs

Note: All pictures shown are for illustration purpose only, actual products may vary due to product enhancement

- INSTALLATION SHOULD ONLY BE CARRIED OUT BY A COMPETENT PERSON

FITTING INSTRUCTIONS

- Fitting the Consumer Unit

- Fit the consumer unit by removing the front cover which are fixed by screws A (fig 1)

- Fix the unit to the required surface with 2 8x1 screws at fixing point B (fig 2)

- Re-fit the front cover.

B

A

B

A

Fig 2

Fig 1

- Fixing the earth bonding cable.

- Connect the earth cable to the fixing point of the chassis, refer to the vehicle information if a hole is required.

- The earth cable is fitted with a 6mm ring terminal.

- Connecting the inlet.

- The unit comes with a pre-wired connector on the orange cable to connect to a pre-wired inlet using the supplied easy connect enclosure (fig 3+4), pre-wired inlets can be purchased from the “Power Part “Range.

Fig 4

Fig 3

- Connecting Sockets and Extensions.

- The unit comes pre-wired with 4 x output feeds (white cable) with connectors which can be attached to various pre-wired sockets and extensions using the supplied light grey easy connect enclosures (fig 5+6), sockets and extensions can be purchased from the “Power Part “Range.

Fig 6

Fig 5

- Fixing the cable to the vehicle.

Note: All pictures shown are for illustration purpose only, actual products may vary due to product enhancement

Use clips (not supplied) to fix the cable to the vehicle at 400mm intervals for vertical runs and 250mm for horizontal runs.

- INSTALLATION SHOULD ONLY BE CARRIED OUT BY A COMPETENT PERSON

Test and Inspection.

- After installation is completed, we recommend an initial inspection is done in accordance with BS7671 by a competent person.

- INSTALLATION SHOULD ONLY BE CARRIED OUT BY A COMPETENT PERSON

We recommend periodic testing of the RCD by pushing the test button the RCD.

Other Products in the 230V Pre-Wired Power Part Range include (entire range is compatible with the pre-wired consumer unit).

- Pre-Wired Inlets.

- Pre-Wired Sockets.

- 230v Output extensions (1M,2M,3M_

- Splitters (allowing to loop from socket to socket)

- Mains Leads (Various Lengths)

When Connecting to the pre-wire range ensure all 230V connections are covered and secured and only accessible via a use of a tool, All electrical systems must be installed and tested by a competent person in accordance with BS7671.

Please ask your dealer about the Power Part range, alternatively please visit the website to see a full list of products WWW.PLSGROUP.CO.UK

Note: All pictures shown are for illustration purpose only, actual products may vary due to product enhancement